Trident 3DImage 9750

High Performance 3D/2D

GUI/Video Accelerator

User's Guide

Version 1.00

NOTICE

The information in this document is subject to change in order to improve reliability, design, or function without prior notice and does not represent a commitment on the part of the company.  In no event will the company be liable for direct, indirect, special, incidental, or consequential damages arising out of the use or the inability to use the product or documentation, even if advised of the possibility of such damages.  No part of this manual may be reproduced or transmitted in any form or by any means without the prior written permission of the company.

Aug 1997, Rev A

Limited Warranty

This product is warranted against defects in materials and workmanship for a period of one year from the date of purchase. During the warranty period, product determined by us to be defective in form or function will be repaired or, at our option, replaced at no charge. This warranty does not apply if the product has been damaged by accident, abuse, misuse, or as a result of service or modification other than by the company.

This warranty is in lieu of any other warranty expressed or implied. In no event shall the company be held liable for incidental or consequential damages, such as lost revenue or lost business opportunities arising from the purchase of this product.

Table of Contents

1. Introduction

1.1 Compatibility

1.2 Check List

1.3 Features

1.3.1 Resolution and Color Selection

1.3.2 3D and 2D GUI Accelerator

1.3.3 VESA DPMS and VESA DDC2B supported

1.3.4 Software MPEG Video Playback

1.3.5 Virtual Screen

1.3.6 DCI and DirectDraw Supported

1.3.7 Geometry Processor

1.3.8 Live Video Capture

1.3.9 TV Out

1.4 Upgrade Option:

1.4.1 Memory Module:

1.4.2 Video Capture and TV Tuner:

2. Adapter Installation

2.1.1 Adapter Layout

2.1.2 Installation Procedures

2.2 Connecting the Display and Optional TV

3. Utilities and Drivers Installation

3.1 Microsoft Windows 95

3.1.1 Welcome screen  from 3DImage 9750's CD

3.1.2 Display Property Configuration:

3.1.3 Microsoft Windows NT 4.0 Driver Installation

3.1.4 Microsoft Windows 3.1x Driver Installation

3.2 Live Video Capture Driver

3.3 DOS Utilities

3.4 Optional TV Out

4. Frequently Asked Questions (FAQ)

5. Troubleshooting

6. Appendix A: Video Mode Table

7. Appendix B: Pinout and Sync Frequencies

7.1 Analog Color Display Pinouts (DB 15)

7.2 Conversion Table:  Pin Adapters

7.3 Analog Video Signals

8. SHIELDED CABLE WARNING

8.1 Notice

8.2 TRADEMARK ACKNOWLEDGMENTS

8.3 Customer Support

I.Introduction

Congratulation on your purchase of the Trident 3DImage 9750 Graphics & Multimedia Accelerator with TV Out. The 3DImage 9750 board uses advanced Trident 3DIMàge975x 3D/2D Graphical User Interface (GUI) accelerator with built-in high-speed 24-bit True Color DAC and video acceleration circuitry and Synchronous Graphics RAM and PCI Bus design. It accelerates graphics display up to 16.7 Million TrueColor at 1024x768 at up to 85Hz vertical refresh rate, plus TV Output and optional TV and Video Input through VMI port with optional TV Tuner and Video Capture card.

3DImage 9750 is Windows 95 Plug and Play compatible. It supports MS DirectX and ActiveMovie.  Free MPEG player software is also included in the CD. It allows all MPEG-1 files and Video CD and other titles to be played back at up to 30 frame-per-second without expensive video hardware.

A.Compatibility

- 486 or Pentium 5x86 system with PCI Local Bus

- Interlace or non-interlace analog monitor

- Multi-sync or PS/2 monitors

- VESA DDC1 and DDC2B monitors detection

- PC97 Graphics Compliant

B.Check List

The package you have purchased should contain the following:

- 3DImage 9750 Graphics & Multimedia Accelerator

- Software and Documentation CD-ROM

- S-Video Cable or RCA Cable (optional)

- 3DImage 9750 Easy setup Guide

If any of these items are missing or damaged, contact your dealer.

Important:  Keep all packaging materials that accompany your adapter in the event you need to return the product.

A.Features

- EDO RAM, 2/4 MB on board, Upgradeable from 2 to 4 MB (SGRAM version optional)

- "Deep Green PC" power management: VBE/PM and  DAC power down

- PCI Specification V2.1 compliant I/O Bus

- VESA Super VGA BIOS Extension Standard, VBE Version 2.0

- Integrated VESA feature connector

- Add-on Video capture, or with TV tuner (optional)

2D Graphics Acceleration

Optimized single cycle 2D graphics engine with a complete feature set in all 8, 16 and True Color modes:

- 256 Raster operation (ROPs)

- 16 bit overlay with scaling

- Trapezoidal fill, solid or pattern

- Polygon fill, solid or pattern

- Line drawing and Clipping

- Stretch and Transparent BLT

- Internal hardware cursor (64x64x2  or 32x32x2)

- Built-in 64x32 pattern register

Direct Draw Support:

- Hardware Page Flipping

- Fast system memory to screen memory BLT

- Color and chroma key support

- Transparent and stretch BLTs

- Overlay support for YUV and RGB

3D Graphics Acceleration

- Balanced the 3D pipeline

- Complete 3D primitive support

- Texture Mapping

- Palletized Texture, 1,2,4,and 8 bit

- Bi-linear filtering

- Mip-Mapping up to 8 levels

- Video texture mapping

- Lighted texturing

- Gouraud Shading for smooth shading

- Alpha blending for transparency effects

- Fog

- Z-buffering at 16/32 bpp

- Provides interface to link to external Geometry co-processor to increase 3D performance

Flicker-Free TV Output

- TV output capability with superior built-in anti-flicker and vertical scaling logic

- Both NTSC/PAL support

- Overscan and underscan timing provided and software switchable

- Built-in gamma correction circuitry to yield better quality on TV Simultaneous display on both TV and CRT with the proper circuitry

- Software programmable to switch between TV and CRT display when in the exclusive display mode

Software MPEG and Video Acceleration

- On-chip Color Space Conversion (CSC), horizontal and vertical interpolated scaling, and overlay control for 30 fps software MPEG/video CODEC acceleration

- Anti-tear support

- Direct Draw /DCI/ENDIVE acceleration with independent x and y minimization/zoom and color space conversion

Hardware Interface to MPEG and TV Video Decoders

- VMI 1.4 compliant hardware interface to MPEG or video decoder for live video input (TV, camera, VCR, and LDP)

- Supports interact/VBI protocol

- Supports planar video format for MPEG1, MPEG2 or Indeo stream

- Dual PCI-bus apertures for graphics and video display memory areas

Video Conferencing

This product it establishes the highest level of 3D performance in PC mass-market. The 3DImage 9750 offers two hardware video windows which reduce CPU loading and increase frame-rate by processing both the local and remote video-conferencing data streams.

Graphic Display Colors and Resolutions*

- 640x480  16, 256, 64K, and 16M colors 60, up to 160Hz

- 800x600  16, 256, 64K, and 16M colors 60, up to 160Hz

- 1024x768  16, 256, 64K, and 16M** colors 43, up to 160Hz

- 1280xl024  16, 256, 64K** colors 43/87I, up to  85Hz

- 1600x1200  16, 256 colors 48/96I, 60Hz

REFRESH RATES IN Hz

Resolution	Colors	87i	96i	60	70	72	75	85	120	160

Standard Modes

320x200	16				

256				

640x200	16				

640x350	16				

Extended Modes

640x480	16			

256									

64K									

16M									

800x600	16			

256									

64K									

16M								

1024x768	16							

256								

64K								

16M							

1280x1024	16							

256							

64K							

1600x1200	16			

256			

*The display resolution and refresh rates available depends on the display mode and monitor capacity.

**4 mega bytes of  display memory required.

Extended Text Display

80 and 132 column text modes in 25, 30, 43, and 60 rows

TV Out Display

NTSC TV: 320x200, 640x200/350/480, 720x350 mono

PAL TV:    All NTSC modes plus 800x600

Compatibility

- 486 or Pentium 5x86 system with PCI Local Bus

- Interlace or non-interlace analog monitor

- Multi-sync or PS/2 monitors

- VESA DDC1 and DDC2B monitors detection

- PC97 Graphics Compliant

Software Driver Support

MS Windows® 3.1x MS Windows®95 MS Windows NT®  4.0

1.Resolution and Color Selection

- Supports up to 1024x768 in 16, 256, 32K, 64K and 16M colors non-interlaced

- Supports 1280x1024 in 16, 256, 32K and 64K colors non-interlaced

- Supports 1600x1200 in 16, 256, 32K and 64K colors non-interlaced

- 80 column text modes in 30, 43 and 60 rows

- 132 column text modes in 25, 30, 43 and 60 rows

1.3D and 2D GUI Accelerator

Accelerates the most frequently used 2D functions in today's graphics-intensive environments plus complete 3D GUI acceleration:

- Complete 3D primitive support

- Texture mapping

- Gouraud Shading for smooth shading

- Alpha blending for transparency effects

- Fog

- Z-buffering at 16/32 bit-per-pixel

- Page flipping for double and triple buffering for smooth animation effects

1.VESA DPMS and VESA DDC2B supported

Supports VESA Display Power management Signaling (DPMS)  which decreases energy consumption when used with a  monitor that meets the VESA standards for power management.  Supports VESA DDC 2b standards for automatically selecting the correct display setup on a monitor that meets the VESA DDC standard.

1.Software MPEG Video Playback

Full-motion software MPEG video playback through 3DImàge975x's fast video accelerator is available with the installation of a software MPEG player.

1.Virtual Screen

The Virtual Screen features takes advantage of un-used display  memory by expanding the display area into the off-screen area; this feature allows the user to make effective use of a display screen larger than the standard 640x480, 800x600 or 1024x768 resolutions.

1.DCI and DirectDraw Supported

Supports DCI for Windows 3.1x and DirectDraw for Windows 95 for enhancement of display quality and playback performance during software MPEG video playback. Both DCI for Windows 3.1x and DirectDraw for Windows 95 are memory manger for video memory.  Using such memory manager for video manger, a program can manipulate video memory with ease, taking full advantage of the blitting and color decompressing capabilities of  3DImage video hardware.

1.Geometry Processor

The optional Geometry Processor off loads the intensive Geometry Calculations from the CPU, thus triples the transform rate during 3D graphics applications to 700K triangles per second.

1.Live Video Capture

Live video capture is the process of accepting video data from an outside source and writing it to the frame buffer from where it can be displayed or stored.  This process is supported by 3DImàge975x through the Feature Connector (FC) and the Video Module Interface (VMI) Host Connector.

1.TV Out

The 3DImage is capable of displaying all standard and some extended VGA video modes on an NTSC or PAL TV.  Resolutions up to 720x480 can be displayed on NTSC TV's; resolutions up to 800x600 can be displayed on PAL TV's.  Extra features are built-in to reduce line flickering and to vertically scale the image for TV display.

A.Upgrade Option:

1.Memory Upgrade:

2MB EDO RAM on your 3DImage board can be upgrade to 4MB by installing 4 pcs of 256x16 50ns EDO RAM.

1.Video Capture and TV Tuner:

The Video Capture card allow your PC to capture live digital video from Video Camera, VCR, and other video source on screen or disk.

The TV Tuner and Video Capture card allow your PC to receive and record video and TV programs (TV antenna or Cable TV wire is needed for receiving TV).

To purchase Video Capture and TV Tuner card, please call your supplier or order through respective manufacture.

I.Adapter Installation

1.Adapter Layout

Figure 2-1.

- CON 3 - Feature Connector (Video Capture)

- RCA Composite Video Out

- S_Video Out

- DB 15 VGA - Video Graphic Adapter

1.Installation Procedures

To install the adapter into your system, follow these steps:

1.Turn OFF all power to your system, including any peripherals (printer, external drives, modem, etc.).

2.Disconnect the power cord and the monitor cable from the back of the computer.

3.Unfasten the cover mounting screws on your system and remove the system cover. Refer to your system user manual for instructions and to determine the location of the mounting screws.

4.Remove any graphics adapter that already exists on your motherboard.  Start by removing the screw that holds the adapter retaining bracket in place (keep this screw, you will need it later).  Then, gently pull straight up on the adapter card itself, and remove it from the motherboard.

5.If appropriate, you can use the expansion slot left vacant by the existing graphics adapter you just removed.  Otherwise, select an appropriate unused PCI bus expansion slot for the new adapter.  Refer to your computer system manual for the location of the PCI bus expansion slots.  Remove the retaining screw that holds the slot cover in place.  Slide the slot cover out and put the screw aside (you will need it to secure the adapter).

If you just removed an existing graphics adapter and are not going to use that expansion slot, you can install the slot cover you just removed from the unused expansion slot to cover the open hole.

6.Install the adapter. To install the adapter in the selected expansion slot, carefully line up the gold-fingered edge connector on the adapter directly above the expansion slot connector on the motherboard.  Then press the adapter into place, completely, using only as much pressure as is safely necessary.  DO NOT USE excessive force. Use the (remaining) screw you removed to secure the adapter retaining bracket in place.

7.Replace the computer cover. Secure the cover with the mounting screws you removed in Step 3.

You have now completed the installation of your new graphics adapter on your system. Before you use the system, however, please refer to the following sections, "Connecting the Display".

A.Connecting the Display and Optional TV

1.Connect your monitor to the adapter standard VGA 15-pin analog connector. Be sure you have the right cable and cable connector. For Pinout information see Appendix B on page 43.

2.If your system is equipped with a sound board, you can also connect a Y-cable for speaker (single male mini stereo phono jack to double male RCA jack) from the Speaker-Out jack of the sound board to the Audio-In jack on your television set.

3.For optional TV connection, please check the back of your TV or VCR for the presence of a S-Video connector. If such a connector is present, you will need a S-Video cable to connect S-Video TV-Out on the adapter to S-Video In on the TV or VCR. If such a connector is not available, a RCA video cable is needed to connect Composite TV-Out on the adapter to Composite Video In on the TV or VCR. S-Video connection is recommended since it provides a higher quality display.

I.Utilities and Drivers Installation

A.Applications on 3DImage 9750 CD

1.Directory's description

3DImage 9750 CD                                        Aug, 97

Directory              Description

-----------------      -------------------------------------------------------------

MANUAL   	User's Manual ( this file)

DOS_UTIL     	DOS application drivers and utility.

DOS Applications Drivers:

AutoCAD/AutoShade, 3D Studio

Utilities:

SVM

SMONITOR

TVDISP.EXE

WIN31          	Windows 3.x DCI Display Drivers .

WIN95 	               Windows 95 Display Drivers .

AVI        	Windows 95 Video Capture Driver Ver

NT40		Windows NT 4.0 Display Drivers

MPEG31        	MPEG Player

MPEG95         	MPEG  95 Player

VFW1_1E         	Microsoft Video for Windows Runtime Version 1.1E

DIRECTX          	Redistribution of MS Windows 95 DirectX 3 API

CLIPS	              Video Sample Files

A.Microsoft Windows 95

- Run the Windows 95 display setup program located in the control panel Select Display icon.

Double click it to bring up Display Property Page.Select “Setting” Tab on the Display Property Page.

- Click “Change Display Type” button from the Display Settings option.

- Select “Change” button from the Display Type options.

- Select “Have “Disk” button from the Select Device options.

- Windows 95 will prompt you for the correct path where the driver is loaded.

- Enter X:\Win95, where X is drive letter of your CD-ROM drive.

- Click OK to start installing the driver files from your CD-ROM drive.

- Choose “3Dimage 9750 Linear Accelerated for PCI (Vx.xx.xx)” from the list of drivers.

- Then follow the procedures provide by Windows 95 to complete installation.

- Restart Windows 95. And installation is complete.

1.Display Property Configuration:

select the "OK" button to close the "Select Device" window and to select the "Color Palette", "Desktop Area", and "Refresh Rate" of your choice under the 3Dimage 9750 accelerated driver (refer to Figure 3-8).

Figure 3-8.

Once the desired color palette (the number of colors), desktop area (resolution) and refresh rate have been chosen, the Windows 95 system will be re-started using the Trident accelerated driver.

1.Microsoft Windows NT 4.0 Driver Installation

1.Select the "Display" icon in control panel and then SELECT the "Settings" page.

2.SELECT "Display Type..." button in the "Settings" page.

3.SELECT "Change..." button from the Adapter type section.

4.Select "Have Disk..." button from the Change Display page.

5.Microsoft Windows NT 4.0 will prompt you for the correct path where the video drivers are located.  Enter the path "X:\NT40" where X: is the CD ROM drive where  the Blaze 3D Ultimate Drivers & Utilities CD has been inserted.

6.If the driver "Trident Video Accelerator" is listed under the Display list, SELECT the "OK" button to continue.

7.Once the driver files are copied, RESTART Microsoft Windows NT 4.0 for the changes to take effect.

8.SELECT the desired color palette (the number of colors), desktop area (resolution), and refresh frequency in the settings page of Display Properties and then SELECT the "Test" button in the same page to determine whether your selection works properly. SELECT "Apply" to active the selected mode.

1.Microsoft Windows 3.1x Driver Installation

The graphic installation program (INSTALL) supports a simple six-step installation procedure for the display driver setup program, the power management program and the UNinstall program.

To use INSTALL, follow the 6 steps below:

- 1.	Ensure that MS Windows 3.1x is up and running properly, 	using the standard VGA driver.

- 2.	Select the MAIN group in Program Manager.

- 3.	Click on FILE or press ALT + F (see Figure 3-9)

Figure 3-9.

- 4.	Click on RUN or press R to select command line.

- 5.	Type in "X:\WIN_31\INSTALL" ("X" is the letter of your CD-ROM drive, typically D: or E:) and then press ENTER.

- 6.	A menu will appear, presenting a choice of Express or Custom Installation.

Express installation is quick and decision free. Display drivers will be copied into the TRIDENT.SGI directory and Utility files will be copied into the TRIDENT.UTL directory. Once all files are copied, a program group called DISPLAY DRIVER AND UTILITIES will be created.

Custom installation allows control over file storage and in what program group the icons are placed. The first dialog box that appears shows the default directory to which the display drivers will be copied. To change the directory name select the default name, delete it and then enter the desired directory name. Once the desired directory name is selected, continue the installation procedure by selecting CONTINUE, or by pressing ENTER. The next dialog box displays a summary of where files are stored. Select CONTINUE to copy the drivers and utilities files. When all files are copied, the program will present a choice of program groups where the icons will be created. Create a new group to place the utility icons or select from pre-existing groups (e.g. main, applications, accessories etc.).

When all necessary files are copied and a group name is selected, the Install program will create three icons:

- a.	Screen Control (Used to configure display drivers).

- b.	DPMS (Used for power management configurations).

- c.	UNinstall (Used to delete the installed TRIDENT drivers).

NOTE:	Different "display driver set" versions 	cannot be installed to the same 	directory name.

a)SCREEN CONTROL

The Screen Control panel contains controls for setting screen resolution, color depth, font size, refresh rates. Not all combinations of screen resolution, color depth, font size and refresh rate are attainable.

Color depths of 16, 256, 64K, or 16.7M colors can be selected by clicking next to the desired option. Color depth determines the number of colors that may be simultaneously displayed on the screen. The selected color depth determines the possible resolutions.

Screen resolutions of 640x480, 800x600, 1024x768, 1280x1024 or 1600x1200 can be selected by clicking next to the available options. The virtual screen size is automatically adjusted to be at least as large as the selected screen resolution.

Available refresh rates are dependent on the selected color depth and resolution. The "Back to Default" option is used to reset the refresh rate to the factory default value in case your monitor does not support a high refresh rate.

(1)Configuring the Display Driver

- Select the color depth first.

- Select the resolution.

- Select the font size (if available as an option).

- Select the refresh rate.

- Click on OK. If the current driver does not support the selected configuration, Windows will have to be restarted.

Figure 3-11.

(1)DDC Monitor Auto Detection

If your monitor is a  "PLUG & PLAY" monitor and is in compliance with VESA DDC2 Standard, the display driver will automatically detect the information provided by the DDC2 complaint monitor.  If the display driver is not able to correctly detect the information provided by the monitor, a message will be displayed that allows you to:

Accept it:  The display driver will save the information from the DDC2 monitor that it was able to detect.

Ignore DDC:  The display driver will not save the information from the DDC2 monitor and this monitor will not be treated as an DDC2 complaint monitor.

Try Again:  The display driver will attempt again to detect the information provided by the DDC2 monitor.

Figure 3-12.

If the display driver is able to detect the monitor successfully, the correct information will be shown once the button "DDC Monitor" in the Advanced page of the Screen Control panel is selected.

Figure 3-13.

A.Live Video Capture Driver

Video Capture is  the process of accepting video data from an source deck, such as TV tuner, VCR, Laser Disc Player or a Camcorder and writing it to the frame buffer.  The data stored in the frame buffer can then be displayed on the monitor or stored in the hard disk drive.  The Trident video capture board for 3Dimage 9750 upgrade is capable of video capturing through the Feature Connector (FC) and the  Video Module Interface (VMI) Host Connector.

Before installation of  Trident AVI drivers, both 3Dimage 9750 display driver and Microsoft Video for Windows 1.1e Runtime for Windows 3.1x must be installed. Skip this step if you use windows 95. Installation procedures for 3Dimage 9750 display driver is in section 3.2.4 of this guide.  Complete Microsoft Video for Windows 1.1e software package can be purchased separately.  While installing Microsoft Video for Windows 1.1e, both Video Playback and Video Tools programs have to be installed to be able to use the Video Capture tools.  Once Video for Windows 1.1e has been installed successfully, the Video for Windows program group will display the programs as shown in Figure 3-14.

Figure 3-14.

The Trident AVI drivers can be installed by following these steps below. Windows 95 User may follow the on-screen instructions.

1.SELECT the MAIN group  in Program Manager.

2.CLICK on FILE or press ALT + F

3.CLICK on RUN or PRESS R to select command line.

4.TYPE in "X:\AVI\SETUP.EXE" ("X" is the letter of your CD-ROM drive, typically D: or E) and then PRESS ENTER .

5.The Trident AVI Drivers Setup screen will appear.  PRESS "Continue" to start installation. (see Figure 3-16).

Figure 3-16.

6.The next screen will place Trident AVI Drivers in the directory "C:\WINDOWS\AVIDRV".  Change the path only if another directory is desired (see Figure 3-17).  Click on "Continue" to complete the installation process of Trident AVI Drivers.

Figure 3-17.

a)Video capture driver usage

This section is for Windows 3.1x. More update the information for Windows 95 can be found with your Video Capture Upgrade kit.

1.Open the "VidCap" program in the Video for Windows 1.1e program group.

2.Turn on the video device that's currently connected to the video-in port of the video capture board and push the "Play" button on the video device to start sending video signals to the video capture board.

3.In the VidCap program window, click on "Overlay Video" under Options (See Figure 3-18).

Figure 3-18.

4.Click on Video Source to set the source of the video signal (see Figure 3-19).

Figure 3-19.

5.Set video source to "Video" to begin displaying video in the overlay window.

a)Video Overlay adjustments

This section is for Windows 3.1x. More update the information for Windows 95 can be found with your Video Capture Upgrade kit.

1. In the VidCap program window, click on " Video Format" under Options to set screen size (see Figure3-20).

Figure 3-20.

2.Select different Image Formats by clicking on one of choices under Image format selections (see Figure 3-21).  While all five formats allow image dimensions of up to full screen size, only "Video Conference YUV 422" image format allows full screen size of 704x576.

Figure 3-21.

3.To configure video or adjust colors, click on " Video Display" under Options (see Figure 3-22).

Figure 3-22.

4.Click on the "Adjust Colors" button to make adjustments of brightness, contrast, saturation and hue.  Click on "Configure Video" button to make adjustments on Overlay Key Control, Display X/Y Offset, and Capture X/Y Offset (see Figure 3-23).

Figure 3-23.

5.Click on "Overlay Key Control" to set VGA Color Key to a color key value other than the default.  Click on "Display X/Y Offset" to adjust the video window within the overlay window.  When the video window is centered, it will completely cover the overlay window (see Figure 3-24).

Figure 3-24.

6.Select 'Capture X/Y Offset" to adjust the color within the video window if the color does not appear correct (see Figure 3-25).

Figure 3-25.

A.DOS Utilities

The Utility Installation program is used to install and retrieve instructions on:

a)Utility programs

b)Non-Windows applications display drivers.

The Utility installation program is run by executing the following steps:

- 1.	Insert the Drivers and Utilities Disk #2 in the floppy drive.

- 2.	Type in "README" at the floppy drive prompt.

- 3.	A numbered list of available destination drives will be

- displayed on the screen.  Select the destination drive by

- typing in the corresponding number; e.g., to select drive C,

- type in "1" (see Figure 3-26).

Figure 3-26.

4.	Files and subdirectories will be expanded into the newly

created directory TVGAUTIL.  A new menu will then be

displayed on the screen, showing a list of on-line instructions

(see Figure 3-27).

Figure 3-27.

- 5.	Selecting A will display the contents of all drivers in the list.

- 6.	Selecting B will display instructions on the available utility 	programs.

- 7.	Selecting C will display instructions on how to install display

- drivers for non-Windows applications.

a)SVM.EXE

SVM is a menu-driven program designed to select and test all video modes available to the adapter.

How To Use SVM

The SVM program can be executed in either of two ways:  by calling up the menu and selecting from the menu choices, or by entering the desired mode directly with a specific command line.

How To Use SVM From The Menu

- 1.	Switch directory to "X:\TVGAUTIL\UTILITY" where X: is

- the drive where the Trident Utility and DOS

- Application driver files have been copied.

- 2.	Type SVM to call up the menu (see Figure 3-28).

Figure 3-28.

The top bar shows the available color depths. This is traversed through by use of the right/left arrow keys.

The program provides all the different resolutions supported by the chip under each color depth. These resolutions are traversed by the up/down arrow keys.The graphics adapter can be tested for each resolution/ mode by first high-lighting the selection (e.g. 1024x768-256 colors as shown in Figure 3-28) by use of the arrow keys, then pressing the F5 key.

The graphics adapter can be run at a selected mode by first selecting the mode and then pressing ENTER.

How to Use SVM From The Command Line

The SVM program may be used to select a mode directly from the command line following two simple steps:

- 1.	Switch directory to "X:\TVGAUTIL\UTILITY" where X: is

- the drive where the Trident Utility and DOS

- Application driver files have been copied.

- 2.	Type in SVM [mode number] and PRESS ENTER.

For example, to run the graphics adapter in mode 62H, the command line would be:

SVM 62 <ENTER>

A. Optional TV Out

In Windows 95, use 'Television' control page and 'Display Device' control page  to select television only features in display properties once the display driver for Windows 95 has been installed.  The 'Television' control page allows the user to select 'Underscan' or 'Overscan', to select 'Interlaced' or 'Non-interlaced',  to adjust the 'Screen Shift' of the television display horizontally and to select the degree of flicker reduction under Advance control button. The 'Display Device' control page allows the user to switch from color monitor to TV as the default display device and vice versa.

In Windows 3.1x, use the Display Control program in Display Driver and Utility Program Group to access TV display features and to switch from color monitor to TV as the default display device and vice versa.  The Display Control program allows the user to select Underscan or Overscan, to select interlaced or non-interlaced, to select degree of flicker reduction and to adjust screen shift both horizontally and vertically.

In DOS, a TSR program "TVDISP" is availabe in the directory of "C:\TVGAUTIL\UTILITY" (where C is the letter of your hard disk drive). This program allows the user to select 'Underscan' or 'Overscan', to select 'Interlaced' or 'Non-interlaced', to switch from color monitor to TV as the default display device and vice versa, to adjust the 'Screen Shift' of the television display both horizontally and vertically,  and to contract or expand the size of screen both horizontally and vertically.  More information about the TVDISP program is available in the REAME.TXT file of this directory.

Conventional broadcast TV or VCR content provides an "Overscanned" viewing area in order to ensure that the entire visible screen is used. The result is that portions of the image along the border are beyond the physical screen. The actual visual area lost is dependent on the individual TV model.  On the other hand, a PC system display content should be within the boundaries of the physically viewable screen. This is called underscan. When the TV is used as the display device in a PC, display hardware needs to control the image generation circuitry to drive the underscan timing to the TV.

Besides generating interlaced timing, hardware may have the option to drive non-interlaced timing to the TV. A non-interlaced display will remove much of the flicker effect but reduce the detail of the image.

The interlaced timing of a TV causes a flickering effect on the screen. Depending on the TV standard and tube phosphor material, the severity of flicker differs from one brand and model of TV to another. Since TV content is almost always full-motion video or animation, flicker is not as noticeable as it is when the screen content is stationary. Flickers are caused by TV low frame rate as well as the interlacing of adjacent scans. This flicker is most obvious when a particular scan contrasts highly with the scans directly above and below, such as a black horizontal line on a white background. This type of flicker can be greatly reduced  by 3DImàge9750's built-in flicker reduction filter.

I.Frequently Asked Questions (FAQ)

Q1  Why do we need 3D graphics capability in our PC?

Answer

3D technology is becoming increasingly important (and common) not only in games, but also in other applications such as VRML, which allows 3D scene descriptions in Web applications. 3D technology is used for image editing, modeling, and an increasing number of in home and business applications. In games, as well as other applications, 3D acceleration not only allows better visual qualities and more realistic scenery attributes than software alone, but it also allows a higher frame rate, which translates into a more interactive experience for the enduser.

Q2  What does "Rendering Engine" mean?

Answer

"Rendering Engine" generically applies to the part of the graphics engine that draws 3D primitives, usually triangles. In most implementations, the rendering engine is responsible for interpolation of edges and "filling in" the triangle.

Q3  What does the set-up engine do in a graphics controller?

Answer

A set-up engine allows drivers to pass triangles in the form of raw vertex information; whereas, most common designs force triangles to be pre-processed for the rendering engine in terms of delta values for edges, color, and texture.

Q4  Why does a 3D graphics chip need to have both a rendering engine and a setup engine?

Answer

Any "3D application", a game, VRML, or modeling package, can benefit from 3D rendering. This is especially true of applications that use texturing extensively, because texturing and texture filtering are very intensive operations at the pixel level in terms of CPU operations and demands for memory bandwidth. Without a set-up engine in a graphics controller, the CPU has to calculate the delta values for edges, color, and textures; the drivers need to handle ten (10) times more extensive data. This results in slower 3D pipeline operations between the CPU and the graphics controller.

Q5  If we use powerful CPUs, such as a Pentium™ 200, can a standard 2D graphics card achieve 3D performance?

Answer

Yes and no. Software rendering can take advantage of "tricks" learned by force of necessity through years of trial and error. With such stratagems, the speed of software rendering for simple scenes can approach that of low-level hardware 3D rendering. On the other hand, as scenes become more complex (or frame sizes become larger), there are conflicts between using the CPU for high-level game logic, geometry, lighting, and rendering, all of which increase their demands. No current CPU or system can perform advanced quality-enhancements (bilinear filtering and alpha blending) in real time. Even general case texture mapping with RGB lighting is too much for the current CPU generation.

Q6  What does "software 3D" mean ?

Answer

Software 3D is generally used to mean using non-specific (2D) hardware in conjunction with the CPU to render for 3D applications. Some of these techniques allow usable 3D applications when high-powered and/or MMX™-equipped CPU's are employed along with special-case software optimization techniques. As stated above, SW 3D can achieve credible results with today's (software optimized) applications, but the rising popularity of good 3D hardware at the consumer price level is inexorably compelling the public to expect hardware level scene enhancements and frame rates.

Q7  What is "SGRAM"?

Answer

Synchronous Graphics Random Access memory (SGRAM) is a new and improved type of memory, custom-designed for graphics use.

Q8  What is the advantage of SGRAM as compared to ordinary DRAM?

Answer

SGRAM is now capable of running at much higher speeds than fast page or EDO DRAM. Also, SGRAM is able to execute a small number of frequently executed operations, such as buffer clears, specific to graphics applications, independently of the controller.

I.Troubleshooting

The following are some recommended steps to take if the GUI accelerator adapter will not boot or operate properly in your system:

1.Ensure that the monitor or TV brightness and contrast controls are properly adjusted.

2.Check to see if your monitor or TV is properly connected to the card.  Be sure your monitor's pin definitions match those of your GUI accelerator card (See Appendix B).  For TV out, ensure that the composite signal is connected to a "Video Input" RCA jack on the TV (or check the S-video connection).  Read the TV owner's manual to select the proper signal jack for the display.

3.Turn the system on and confirm that the power supply is operating properly; i.e., that the fan operates and the system power light turns on.

4.Check to see if the card is firmly seated in its PCI bus expansion slot. It should not be making contact with any other cards in the system.

Note: Turn the system off before adjusting the card.

Problem:	Windows hangs up during or after installing a driver.

Solution A:	Reread installation procedures to be sure you have installed the drivers correctly.

Problem:	Windows color palette does not look right or colors changing.

Solution:	Most likely a defective RAMDAC, memory chip, clock chip, or crystal. Contact your dealer to have the problem repaired.

Problem:	Can't display certain modes.

Solution A:	Run the SVM program (See the  User's Guide for more information on the SVM program). If the SVM program fails, go to Solutions B, and C.

Solution B:	Check to see that there is enough memory on the GUI accelerator to run this mode. For example, to run display mode 79H (1024x768-64K colors, refer to the tables in Section 2), 2 MB of display memory is required.

Solution C:	If Solutions A, or B do not resolve this problem, it may be hardware related.  Check the specifications of the monitor.

I.Appendix A: Video Mode Table

The adapter's video modes include all of the following:

Table 6-1:  Standard vga Mode Support

Mode #	Resolution -Colors	Horz KHz	Vert Hz	Mem Req	Text Res	Mode Type	TV Out

0h,1h	320x200-16	31.4	70	512K	40x25	Text	N,P

2h,3h	640x400-16	31.4	70	512K	80x25	Text	N,P

4h,5h	320x200-4	31.4	70	512K	40x25	Graph	N,P

6h	640x200-2	31.4	70	512K	80x25	Graph	N,P

7h	720x350-Mono	31.5	70	512K	80x25	Text	N,P

Dh	320x200-16	31.4	70	512K	40x25	Graph	N,P

Eh	640x200-16	31.4	70	512K	80x25	Graph	N,P

Fh	640x350-2	31.4	70	512K	80x25	Graph	N,P

10h	640x350-16	31.4	70	512K	80x25	Graph	N,P

11h	640x480-2	31.4	60	512K	80x30	Graph	N,P

12h	640x480-16	31.4	60	512K	80x30	Graph	N,P

13h	320x200-256	31.4	70	512K	40x25	Graph	N,P

Table 6-2:  extended VGA mode support

Mode #	Resolution -Colors	Horz KHz	Vert Hz	Mem Req	Text Res	Mode Type	TV Out

2Ch	320x200-256	31.6	70	512K	40x12	Graph	N,P

2Dh_4	320x240-256	43.3	85	512K	40x15	Graph

2Dh_3	320x240-256	37.5	75	512K	40x15	Graph

2Dh_2	320x240-256	37.9	72	512K	40x15	Graph

2Dh_1	320x240-256	31.5	60	512K	40x15	Graph	N,P

2Eh_3	400x300-256	53.7	85	512K	50x18	Graph

2Eh_2	400x300-256	46.9	75	512K	50x18	Graph

2Eh_1	400x300-256	37.9	60	512K	50x18	Graph	P

3Bh	320x200-16M	31.6	70	512K	40x12	Graph	N,P

3Ch_4	320x240-16M	43.3	85	512K	40x15	Graph

3Ch_3	320x240-16M	37.5	75	512K	40x15	Graph

3Ch_2	320x240-16M	37.9	72	512K	40x15	Graph

3Ch_0	320x240-16M	31.5	60	512K	40x15	Graph	N,P

3Dh_3	400x300-16M	53.7	85	512K	50x18	Graph

3Dh_2	400x300-16M	46.9	75	512K	50x18	Graph

3Dh_1	400x300-16M	37.9	60	512K	50x18	Graph	P

42/3h	320x200-32K/64K	31.6	70	512K	40x12	Graph	N,P

44/5h_4	320x240-32K/64K	43.3	85	512K	40x15	Graph

44/5h_3	320x240-32K/64K	37.5	75	512K	40x15	Graph

44/5h_2	320x240-32K/64K	37.9	72	512K	40x15	Graph

44/5h_1	320x240-32K/64K	31.5	60	512K	40x15	Graph	N,P

46/7h_4	400x300-32K/64K	53.7	85	512K	50x18	Graph

46/7h_3	400x300-32K/64K	46.9	75	512K	50x18	Graph

46/7h_2	400x300-32K/64K	37.9	60	512K	50x18	Graph	P

50h	640x480-16	31.5	60	512K	80x43	Text	N,P

51h	640x473-16	31.5	60	512K	80x43	Text

52h	640x480-16	31.5	60	512K	80x60	Text	N,P

53h	1056x350-16	31.3	70	512K	132x25	Text

54h	1056x480-16	31.3	60	512K	132x30	Text

55h	1056x473-16	31.3	60	512K	132x43	Text

56h	1056x480-16	31.3	60	512K	132x60	Text

57h	1188x350-16	31.3	70	512K	132x25	Text

58h	1188x480-16	31.3	60	512K	132x30	Text

59h	1188x473-16	31.3	60	512K	132x43	Text

5Ah	1188x480-16	31.3	60	512K	132x60	Text

5Bh_3	800x600-16	53.7	85	512K	100x75	Graph

5Bh_2	800x600-16	46.8	75	512K	100x75	Graph

5Bh_1	800x600-16	37.8	60	512K	100x75	Graph	P

5Ch	640x400-256	31.6	70	512K	80x25	Graph	N,P

5Dh_4	640x480-256	43.2	85	512K	80x30	Graph

5Dh_3	640x480-256	37.5	75	512K	80x30	Graph

5Dh_2	640x480-256	37.8	72	512K	80x30	Graph

5Dh_1	640x480-256	31.4	60	512K	80x30	Graph	N,P

5Eh_3	800x600-256	53.7	85	512K	100x37	Graph

5Eh_2	800x600-256	46.8	75	512K	100x37	Graph

5Eh_1	800x600-256	37.8	60	512K	100x37	Graph	P

5Fh_5	1024x768-16	68.7	85	512K	128x48	Graph

5Fh_4	1024x768-16	60.4	75	512K	128x48	Graph

5Fh_3	1024x768-16	56.4	70	512K	128x48	Graph

5Fh_2	1024x768-16	48.5	60	512K	128x48	Graph

5Fh_1	1024x768-16	35.5	87i	512K	128x48	Graph

62h_5	1024x768-256	68.7	85	1M	128x48	Graph

62h_4	1024x768-256	60.0	75	1M	128x48	Graph

62h_3	1024x768-256	56.4	70	1M	128x48	Graph

62h_2	1024x768-256	48.3	60	1M	128x48	Graph

62h_1	1024x768-256	35.5	87i	1M	 128x48	Graph

63h_3	1280x1024-16	80.0	75	1M	160x64	Graph

63h_2	1280x1024-16	63.9	60	1M	160x64	Graph

63h_1	1280x1024-16	46.4	87i	1M	160x64	Graph

64h_4	1280x1024-256	91.1	85	2M	160x64	Graph

64h_3	1280x1024-256	80.0	75	2M	160x64	Graph

64h_2	1280x1024-256	63.9	60	2M	160x64	Graph

64h_1	1280x1024-256	46.4	87i	2M	160x64	Graph

65h_2	1600x1200-16	75	60	1M	200x75	Graph

65h_1	1600x1200-16	62.5	96i	1M	200x75	Graph

66h_4	1600x1200-256	106.3	85	2M	200x75	Graph

66h_3	1600x1200-256	93.8	75	2M	200x75	Graph

66h_2	1600x1200-256	75	60	2M	200x75	Graph

66h_1	1600x1200-256	62.5	96i	2M	200x75	Graph

6Ah_11	800x600-16	37.8	60	512K	100x75	Graph

6Bh	640x400-16M	31.6	70	2M	80x25	Graph	N,P

6Ch_4	640x480-16M	43.2	85	2M	80x30	Graph

6Ch_3	640x480-16M	37.5	75	2M	80x30	Graph

6Ch_2	640x480-16M	37.8	72	2M	80x30	Graph

6Ch_0	640x480-16M	31.4	60	2M	80x30	Graph	N,P

6Dh_3	800x600-16M	53.7	85	2M	100x37	Graph

6Dh_2	800x600-16M	46.8	75	2M	100x37	Graph

6Dh_1	800x600-16M	37.8	60	2M	100x37	Graph	P

66h_2	1600x1200-256	75	60	2M	200x75	Graph

6Eh_5	1024x768-16M	68.7	85	4M	128x48	Graph

6Eh_4	1024x768-16M	60.0	75	4M	128x48	Graph

6Eh_3	1024x768-16M	56.4	70	4M	128x48	Graph

6Eh_2	1024x768-16M	48.3	60	4M	128x48	Graph

6Eh_1	1024x768-16M	35.5	87i	4M	128x48	Graph

72/3h	640x400-32K/64K	31.6	70	1M	80x25	Graph

74/5h_4	640x480-32K/64K	43.2	85	1M	80x30	Graph

74/5h_3	640x480-32K/64K	37.5	75	1M	80x30	Graph

74/5h_2	640x480-32K/64K	37.8	72	 1M	80x30	Graph

74/5h_1	640x480-32K/64K	31.4	60	1M	80x30	Graph	N,P

76/7h _4	800x600-32K/64K	53.7	85	1M	100x37	Graph

76/7h_3	800x600-32K/64K	46.8	75	1M	100x37	Graph

76/7h_2	800x600-32K/64K	37.8	60	1M	100x37	Graph	P

78/9h_5	1024x768-32K/64K	68.7	85	2M	128x48	Graph

78/9h_4	1024x768-32K/64K	60.0	75	2M	128x48	Graph

78/9h_3	1024x768-32K/64K	56.4	70	2M	128x48	Graph

78/9h_2	1024x768-32K/64K	48.3	60	2M	128x48	Graph

78/9h_1	1024x768-32K/64K	35.5	87i	2M	128x48	Graph

7A/Bh_3	1280x1024-32K/64K	80	75	4M	160x64	Graph

7A/Bh_2	1280x1024-32K/64K	63.9	60	4M	160x64	Graph

7A/Bh_1	1280x1024-32K/64K	46.4	87i	4M	160x64	Graph

7C/Dh_4	1600x1200-32K/64K	106.3	85	4M	160x64	Graph

7C/Dh_3	1600x1200-32K/64K	93.8	75	4M	160x64	Graph

7C/Dh_2	1600x1200-32K/64K	75.0	60	4M	160x64	Graph

7C/Dh_1	1600x1200-32K/64K	62.5	96i	4M	160x64	Graph

NOTES:

1.VESA mode.  Same as 5Bh_1.

2.The "i" in the vertical frequency column denotes "interlaced".  The "N"

and "P" in the TV Out column denote "NTSC" and "PAL", respectively.

I.Appendix B: Pinout and Sync Frequencies

A.Analog Color Display Pinouts (DB 15)

Pin	Function

1	Red Video1

2	Green Video1

3	Blue Video1

4	Not Used

5	Ground

6	Red Return (ground)

7	Green Return (ground)

8	Blue Return (ground)

9	Vcc (+5v DDC Power)

10	Sync Return (ground)

11	Monitor ID (not used)

12	SDA (DDC support)

13	Horizontal Sync

14	Vertical Sync

15	SCL (DDC support)

Note:	Analog monochrome type monitors use green video for all video input and ignore red and blue video.

A.Conversion Table:  Pin Adapters

If you will be using a 9-to-15 pin adapter cable to link your 9 pin monitor connector to the 15 pin accelerator card connector, check Table 7-2 carefully before you install the cable.  The 9-to-15 pin adapter cables are available from a variety of sources, but they need to match the specifications in Table 7-2 to work properly with your new card.

The adapter cable requires a D-shaped 9 pin female connector and a D-shaped 15 pin male connector.

Table 7-2.  9-to-15 Pin Conversion Table

9 pin Signals	Pin No.	15 pin Signals	Pin no.

Red	1	Red	1

Green	2	Green	2

Blue	3	Blue	3

Horz Sync	4	Horz Sync	13

Vert Sync	5	Vert Sync	14

Red Ground	6	Return Red	6

Green Ground	7	Return Green	7

Blue Ground	8	Return Blue	8

Sync Ground	9	Digital Ground	10

Ground	5

A.Analog Video Signals

Black Level = 0 V

Full Intensity (White) Level = +0.7 V

I.SHIELDED CABLE WARNING

This equipment has been tested and found to comply with the limits for a Class B digital device, pursuant to Part 15 of the FCC Rules. These limits are designed to provide reasonable protection against harmful interference in a residential installation. This equipment generates, uses and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications. However, there is no guarantee that interference will not occur in a particular installation. If this equipment does cause interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

Reorient or re-locate the receiving antenna.

Increase the separation between the equipment and the receiver.

Connect the equipment into an outlet on a circuit different from that

to which the receiver is connected.

Consult an experienced radio/TV technician for help.

The user may find the following booklet prepared by the Federal Communications Commission helpful:

"How to Identify and Resolve Radio/TV Interference Problems".

This booklet is available from the U.S. Government Printing Office, Washington, DC 20402, Stock No. 004-000-00345-4.

This device complies with Part 15 of the FCC Rules. Operation is subject to the following conditions: (1) this device may not cause harmful interference, and (2) this device must accept any interference received, including interference that may cause undesired operation,

"SHIELD INTERFERENCE CABLE(S) MUST BE USED ACCORDING TO FCC 15.27©."

CAUTION: Changes or modifications not expressly approved by the Manufacturer could void your authority to operate this equipment in accordance with FCC rules and regulations.

A.Notice

The information in this document is subject to change in order to improve reliability, design, or function without prior notice and does not represent a commitment on the part of this company.  The information in this manual is believed to be accurate. However, Jaton Corporation assumes no responsibility for any inaccuracies that may be contained in this manual. In no event will we be liable for direct, indirect, special, incidental, or consequential damages arising out of the use or the inability to use the product or documentation, even if advised of the possibility of such damages.  No part of this reference manual may be reproduced or transmitted in any form or by any means without the prior written permission.

May, 1997

A.TRADEMARK ACKNOWLEDGMENTS

Trident Microsystems, Inc. 1997. All rights reserved.

Trident Microsystems, Inc. 3DImàge975x™ User's Guide

Trident Microsystems, Inc. assumes no responsibility for the use of any circuit other than circuits embodied in an Trident Microsystems, Inc. product.  No other circuit patent licenses are implied.

AutoCAD and Autoshade are trademarks of Autodesk, Inc.

IBM is a registered trademark and PS/2 and OS/2 are trademarks of International Business Machines Corp.

Microsoft, Microsoft Windows, and Microsoft Word are registered trademarks of Microsoft Corp.

Pentium is a trademark of Intel Corp.

RAMDAC is a trademark of Brooktree Corporation.

3DImàge9750, 3DImàge9753 and TVGA are registered trademarks of Trident Microsystems, Inc.

VESA is a trademark of Video Electronics Standards Association.

All other product names or trademarks are property of their respective owners.

Copyright protection claimed includes all forms and matters of copyrightable material and information now allowed by statutory or judicial law or hereinaffer granted, including without limitation, material generated from the software programs which are displayed on the screen such as icons, screen display looks, etc.  Reproduction or disassembly of embedded computer programs or algorithms prohibited.

A.Customer Support

In the event you have a technical problem with this product, please read  the README files in the driver disks. Updated drivers are available through BBS. Have following information handy when you contact technical support:

- Name of the product.

- Software Driver and Version.

- System Information, such as CPU speed, BIOS version, Monitor Specification, etc.

- Description of the problems including any error messages.