# Maintenance and Installation

This chapter describes opening Intergraph’s desktop and deskside 3D graphics workstations to replace a GLZ1 board.  It also describes installing a dual screen option in the desktop and deskside workstations.

## Static Precautions

Observe the following precautions when working with the internal components of the system.

- Avoid servicing the system in areas with electrostatic problems.  Rugs and carpets, for example, cause static build-up.

- Handle all printed circuit boards as little as possible and by the edges only.  Leave the boards in their protective packaging until you install them.

CAUTION

Discharge personal static charge by touching a bare metal surface on the workstation before handling printed circuit boards.

## Opening the Desktop Workstation

WARNING

Before opening the workstation, turn off the system power.  Leave the power cord connected to maintain ground.  Use caution when removing the cover to avoid injury.

ä To open the desktop workstation:

1.	Pull out the spring-lock plunger on the back panel and turn it clockwise until it locks in the open position.

4715A

ISA I/O

Panel

Spring-lock

Plunger

Figure 2-1.  Desktop Top Cover Removal

2.	Lift up the top cover above the ISA I/O panel and pull up and back to remove the top cover.

3.	Attach an antistatic wrist strap to a non-painted, metallic surface on the workstation or to the ground loop (formed in the metal) above the ISA I/O panel.  Ensure that the elastic loop in the strap snugly contacts your bare wrist.

## Replacing GLZ1 Board in the Desktop

This section describes replacing a graphics board in a desktop workstation.  For single monitor workstations, a GLZ1 board should reside in PCI slot 2.  Refer to Figure 2-2 for board and PCI slot locations.

ä To replace a GLZ1 board in the desktop workstation:

1.	Disconnect the cables from the Video and VGA ports on the back panel of the base unit.

2.	Using a quarter-inch nutdriver, remove the screw from the currently installed GLZ1 board.  Retain the screw.

3.	Remove the GLZ1 board from the workstation.

4.	Align the card guides on the new GLZ1 board and install it into PCI slot 2.

5.	Install the screw previously removed to secure the GLZ1 board to the workstation.

## Installing a Dual Screen Option in the Desktop

For dual monitors, two GLZ1 boards should reside in PCI Slots 1 and 2.  The Intergraph GLZ1 dual screen option kit provides the following components to upgrade from a single screen to a dual screen configuration.  Verify that you have the following items.

- GLZ1 board

- Diskette containing the Intergraph GL display driver software

- GLZ1 Hardware User’s Guide

- Clock sync cable

- FIFO status cable  (only used if Geometry Accelerator option board is present)

- Antistatic wrist strap

If an item is missing or damaged, notify Intergraph Customer Response Center at 1-800-633-7248 immediately.

ä To install the second GLZ1 board for dual monitors:

1.	Using a quarter-inch nutdriver, remove the blanking plate from slot 1.  Retain the screw.

2.	Install the second GLZ1 board into PCI slot 1.

4694

Slot 1

Slot 2

GLZ1 Board

Screw

Figure 2-2.  GLZ1 Board for Desktop, Dual Monitors

3.	Install the screws to secure the GLZ1 boards to the workstation.

CAUTION

The clock sync cable installs only one way.  Do not force the keyed connector.  Ensure pin 1 of both cable connectors engages pin 1 on both board connectors.  The black wire connects to pin 1.

4.	Install the clock sync cable between the two GLZ1 boards as shown in the following figure.

4695

Clock Sync Cable

Secondary

GLZ1

Board

Figure 2-3.  Clock Sync Cable Installation

## Completing the Installation for the Desktop

Follow these steps to reassemble the workstation and reconnect the monitor.

ä To complete the installation:

1.	Replace the top cover on the workstation by aligning the tabs on the front of the top cover with the notches behind the faceplate.

2.	Lower the back of the top cover and slide it into place.

3.	Turn the spring-lock plunger to lock the top cover to the base unit.

4.	Connect the VGA Loopback cable and monitor cable for single monitor configurations as follows:

VGA Loopback:  Connect the VGA Loopback cable from the VGA Out port to the VGA In port as shown in Figure 2-4.

Single monitor:  Connect the monitor cable to the Video Out port in PCI slot 2.

4732A

VGA In PortVGA Out

Port

VGA Loopback Cable

Video Out Port

Figure 2-4.  Single Monitor Cable Connections

5.	Connect the VGA Loopback cable and monitor cables for dual monitor configurations as follows:

VGA Loopback:  Connect the VGA Loopback cable from the VGA Out port to the VGA In port as shown in Figure 2-5.

Primary monitor:  Connect the monitor cable to the Video Out port in PCI slot 1.

Secondary monitor:  Connect the monitor cable to the Video Out port in PCI slot 2.

4765

VGA In PortVGA Out

Port

VGA Loopback Cable

Video Out Port

(Secondary)

Video Out Port

(Primary)

Figure 2-5.  Dual Monitor Cable Connections

6.	If you need to configure the GL display driver, refer to “Configuring the Video Display” in Chapter 1.

## Opening the Deskside Workstation

WARNING

Before opening the workstation, turn off the system power.  Leave the power cord connected to maintain ground.  Use caution when removing the cover to avoid injury.

ä To open the deskside workstation:

1.	Using a quarter-inch nutdriver, remove the screw from the top cover.  Retain the screw.

4136B + MOD (deleted SCSI connector)

Screw

Figure 2-6.  Deskside Top Cover Removal

2.	Remove the top cover by sliding it toward the back of the workstation.

3.	Remove the footstands.

4.	Remove the two screws from the top of the left side panel.

4636

Screws

Figure 2-7.  Deskside Left Side Panel Removal

5.	Remove the left side panel by grasping the tab between the screw holes.  Then, slide the panel up and out from the workstation.

6.	Lay the workstation on its right side.

7.	Remove the three screws from the PCI access panel on the bottom of the workstation.

4696

Screw

PCI Access

Panel

Screws

Figure 2-8.  Deskside PCI Access Panel Removal

8.	Remove the PCI access panel by sliding it toward the back of the workstation.

9.	Attach an antistatic wrist strap to a non-painted, metallic surface on the workstation.  Ensure that the elastic loop in the strap snugly contacts your bare wrist.

## Replacing GLZ1 Board in the Deskside

This section describes replacing a GLZ1 board in a deskside workstation.  For single monitor workstations, a GLZ1 board should reside in PCI slot 2.

ä To replace a GLZ1 board in the deskside workstation:

1.	Disconnect the cable(s) from the Video and VGA ports on the back panel of the base unit.

2.	If replacing a secondary GLZ1 board in a dual screen configuration, disconnect the clock sync cable.

4695

Clock Sync Cable

Secondary

GLZ1

Board

Figure 2-9.  Clock Sync Cable Removal

3.	Using a quarter-inch nutdriver, remove the screw from the GLZ1 board.  Retain the screw.

4.	Remove the GLZ1 board from the workstation.

5.	Install the new GLZ1 board into the slot from which the previously installed board was removed.

Note:	In a dual screen configuration, the primary GLZ1 board resides in PCI slot 2 and the secondary GLZ1 board in PCI slot 3.  Refer to Figure 2-10 for board and PCI slot location.

6.	Secure the board to the base unit using the screw removed previously.

7.	If you replaced a GLZ1 board in a dual screen configuration, install the clock sync cable.

CAUTION

The clock sync cable installs only one way.  Do not force the keyed connector.  Ensure pin 1 of both cable connectors engages pin 1 on both board connectors.  The black wire connects to pin 1.

## Installing a Dual Screen Option in the Deskside

The Intergraph GLZ1 dual screen option kit provides the following components to upgrade from a single screen to a dual screen configuration.  Verify that you have the following items.

- GLZ1 board

- Diskette containing the Intergraph GL display driver software

- GLZ1 Hardware User’s Guide

- Clock sync cable

- FIFO status cable (only used if Geometry Accelerator option board is present)

- Antistatic wrist strap

If an item is missing or damaged, notify Intergraph Customer Response Center at 1-800-633-7248 immediately.

ä To install a dual screen option in the deskside:

1.	Using a quarter-inch nutdriver, remove the blanking plate from PCI
slot 3.  Retain the screws.  Refer to Figure 2-10 for board and PCI slot locations.

2.	Install the second GLZ1 board into PCI slot 3.

4632

PCI Slot 1

PCI Slot 2

PCI Slot 3

PCI Slot 6

PCI Slot 5

PCI Slot 4

Figure 2-10.  Deskside PCI Slots

3.	Install the screw to secure the GLZ1 board to the workstation.

4.	Install the clock sync cable between the two GLZ1 boards.  Refer to Figure 2-11.

CAUTION

The clock sync cable installs only one way.  Do not force the keyed connector.  Ensure pin 1 of both cable connectors engages pin 1 on both board connectors.  The black wire connects to pin 1.

4695

Clock Sync Cable

Secondary

GLZ1

Board

Figure 2-11.  Clock Sync Cable Installation

## Completing the Installation for the Deskside

Follow these steps to reassemble the workstation and reconnect the monitor.

ä To complete the installation:

1.	Install the PCI access panel on the workstation.

2.	Raise the workstation to its normal standing position.

3.	Install the side panel of the workstation.

4.	Install the footstands.

CAUTION

To avoid injury or damage to the equipment, replace the stabilizing footstands before operating the unit.

5.	Install the top cover of the workstation.

6.	Connect the VGA Loopback cable and monitor cable for single monitor configurations as follows:

VGA Loopback:  Connect the VGA Loopback cable from the VGA Out port to the VGA In port as shown in Figure 2-12.

Single monitor:  Connect the monitor cable to the Video Out port in PCI slot 2.

4709B

Video

Out Port

VGA Out Port          VGA In Port

VGA Loopback

Cable

Figure 2-12.  Single Monitor Cable Connections

7.	Connect the VGA Loopback cable and monitor cables for dual monitor configurations as follows:

VGA Loopback:  Connect the VGA Loopback cable from the VGA Out port to the VGA In port as shown in Figure 2-13.

Primary monitor:  Connect the monitor cable to the Video Out port in PCI slot 2.

Secondary monitor:  Connect the monitor cable to the Video Out port in PCI slot 3.

4764

Video

Out Port

(Secondary)

Video

Out Port

(Primary)

VGA Out Port   VGA In Port

VGA Loopback

Cable

Figure 2-13.  Dual Monitor Cable Connections

8.	If you need to configure the GL display driver, refer to “Configuring the Video Display” in Chapter 1.